ARTICLE REVIEW

5G / 6G Communication Systems

Name: RAMANUJAM.B

Register Number: 192512349

Course: 5G/6G Communication Systems

Course Code: ECA5606

FACULTY: Dr. KALAIARASI K

Slot: C

1. Robotic Automation in Industry 4.0 Using 5G Communication

Industry 4.0 represents the convergence of automation, data exchange, and intelligent systems into a unified framework aimed at building self-optimizing "smart factories." Robotic automation lies at the heart of this transformation, but conventional wired or Wi-Fi-based communication imposes limits on robot mobility, reconfigurability, and real-time responsiveness. Fifth-generation (5G) mobile communication addresses these constraints through ultra-reliable low-latency communication (URLLC), massive machine-type communication (mMTC), and enhanced mobile broadband (eMBB), making it a natural connectivity backbone for industrial robots, Automated Guided Vehicles (AGVs), and collaborative robots (cobots) operating on the modern factory floor.

A 5G-enabled robotic automation framework is typically organized into four layers. The device layer comprises industrial robots, AGVs, sensors, and actuators. The connectivity layer consists of the 5G Radio Access Network (RAN), small cells, device-to-device (D2D) communication, and network slicing, which together provide deterministic, low-latency links between machines. The edge/cloud layer offloads computation-intensive tasks such as computer vision, path planning, and AI-based decision-making away from the robot controller, reducing onboard complexity while enabling real-time closed-loop control at latencies as low as 1 millisecond. Finally, the application layer supports functions such as predictive maintenance, automated quality inspection, and human-robot collaboration.

Network slicing allows operators to create dedicated virtual networks for robotics, monitoring, and control traffic, each with guaranteed Quality of Service (QoS), ensuring that safety-critical control signals are never delayed by less time-sensitive data such as video monitoring. This capability, combined with the removal of physical cabling, gives manufacturers the flexibility to reposition robots and reconfigure production lines on demand — a core requirement of agile, mass-customized manufacturing under Industry 4.0.

Despite these advances, several challenges remain open. Achieving guaranteed deterministic latency at scale requires tighter integration between 5G and Time-Sensitive Networking (TSN) standards. Private 5G network deployment carries significant infrastructure cost, and wireless industrial control systems introduce new cybersecurity risks that must be addressed through robust authentication and encryption. Interoperability between legacy factory equipment and modern 5G-based systems, along with spectrum management in dense industrial environments, also remain active areas of research. Addressing these challenges will be essential to fully realizing the potential of 5G-enabled robotic automation as manufacturing evolves toward the human-centric paradigm of Industry 5.0.

REFERENCE BASE PAPER

Primary Reference

Markus Aleksy, Fan Dai, Nima Enayati, Peter Rost, and Guillermo Pocovi, “Utilizing 5G in Industrial Robotic Applications,” 2019 7th International Conference on Future Internet of Things and Cloud (FiCloud), Istanbul, Turkey, pp. 278–284, 2019.

DOI: 10.1109/FiCloud.2019.00046

This paper describes why 5G technology can make a significant contribution toward enabling new types of industrial robotics applications. The authors show that sensor-based real-time control increasingly requires computing power beyond what an onboard robot controller can provide, and that offloading this computation to an external computer over conventional wireless links introduces communication delays that make real-time control difficult. They present several industrial use cases and simulation results demonstrating that a 5G network, combined with a local factory cloud, can bridge this latency gap and support flexible, wirelessly connected robotic motion control. It serves as an excellent foundational reference for understanding how 5G communication underpins robotic automation in Industry 4.0.

Additional Supporting References

1. S. K. Rao and R. Prasad, “Impact of 5G Technologies on Industry 4.0,” Wireless Personal Communications, vol. 100, no. 1, pp. 145–159, 2018.

2. 3GPP, “Technical Specification Group Services and System Aspects; Service Requirements for Cyber-Physical Control Applications in Vertical Domains,” 3GPP TR 22.104, V16.1.0, 2019.

3. “A Survey of 5G-Based Positioning for Industry 4.0: State of the Art and Enhanced Techniques,” arXiv:2409.13308.

4. “5G-Cloud-based Real-Time Robotic Part Repairing for Advanced Manufacturing via Computer Vision,” ScienceDirect, 2024.

5. F. Ansah, M. Majumder, H. de Meer, and J. Jasperneite, “Network Slicing: An Industry Perspective,” 2019 24th IEEE International Conference on Emerging Technologies and Factory Automation (ETFA), pp. 1367–1370, 2019.